KnowYourCodesMB.ca

Compliance Tool — Project Scope

Phase 1 Addition — Codes Acceleration Fund Project

| Prepared by | Prepared for | Date | Version |
| --- | --- | --- | --- |
| Honest Agency | Engineers Geoscientists Manitoba (EngGeoMB) | October 06, 2026 | 1.0 |

1. OVERVIEW AND RATIONALE

KnowYourCodesMB.ca is Manitoba's plain-language energy code information hub, developed by Engineers Geoscientists Manitoba (EngGeoMB) with funding from the federal Codes Acceleration Fund (CAF). The site provides guidance on Manitoba's adoption of Tier 3 (residential, NBC 2025 Part 9) and Tier 2 (commercial, NECB 2025), effective January 1, 2028.

This document scopes an interactive compliance tool to be built as part of Phase 1 of the project. The tool allows builders, designers, and building officials to work through the three compliance paths — prescriptive, trade-off, and performance — for both residential and commercial buildings, and determine whether a proposed building design meets Manitoba's adopted energy code tier.

Why a tool?  The content pages on KnowYourCodesMB.ca explain the code. The tool helps users apply it. Together they serve the full user journey: understand the code, then verify compliance. A tool also dramatically increases the site's ongoing utility and return visits.

2. Manitoba Code Context

|  | Residential | Commercial |
| --- | --- | --- |
| Code | NBC 2025 Part 9 | NECB 2025 |
| Adopted Tier | Tier 3 | Tier 2 |
| Effective Date | January 1, 2028 | January 1, 2028 |
| Codes Adopted | June 22, 2027 | June 22, 2027 |
| Climate Zones | 7A, 7B, 8 | 7A, 7B, 8 |

2. USERS

The tool is designed primarily for builders and designers, with a role-adapted experience for engineers, architects, and building officials. Role selection at entry adjusts language and detail level — the underlying calculations and code references are identical.

Primary Users

Builder / Developer

Knows what they're building, less familiar with code specifics. Needs plain-language guidance, clear pass/fail results, and actionable suggestions when not compliant.

Designer / Architect

Familiar with RSI, U-values, and code structure. Wants a fast, accurate compliance check without extensive hand-holding. May be working on multiple projects.

Secondary Users

Engineer

High code literacy. Uses the tool for verification rather than navigation. Expects precise values and direct code citations.

Building Official

Review mode: inputs a submitted set of values to verify compliance rather than designing. Same calculations, different framing.

3. COMPLIANCE PATHS IN SCOPE

The tool covers all three code-defined compliance paths for both residential and commercial buildings.

| Path | Residential | Commercial | Tool Approach |
| --- | --- | --- | --- |
| Prescriptive | NBC 9.36.2–9.36.4 (base) + 9.36.9 (tiered points) | NECB Parts 3–7 | Full calculation tool |
| Trade-off | NBC 9.36.2.11 (simple trade-off within envelope) | NECB Section 3.3 | Full calculation tool |
| Performance | NBC 9.36.7 (HOT2000 modelling) | NECB Part 8 + Part 10 | Screening estimator + referral to modeller |

Performance path note  The residential performance path requires HOT2000 software per NBC 9.36.7 — this cannot be replicated in a web tool for permit purposes. The tool provides a simplified screening estimate to help users understand their likely tier before engaging a certified energy modeller. Output is clearly labelled as an estimate, not a permit-ready result. Same approach for commercial performance path (NECB Part 8).

4. RESIDENTIAL TOOL FLOW

NBC 2025 Part 9 — Manitoba adopts Tier 3 (20 conservation points minimum, 5 from envelope measures).

Step 1 — Location

Dropdown of 24 Manitoba cities/towns

Auto-assigns HDD value and climate zone (7A, 7B, or 8)

Small Manitoba map highlights the zone for context

Manual HDD entry option for locations not listed

Step 2 — Building Basics

Building type: detached house / semi-detached / row house / small multi-unit

Number of dwelling units

Total conditioned volume (m³) — with plain-language explainer and diagram

HRV or ERV included? (yes/no) — determines which base tables apply

Step 3 — Choose Compliance Path

Three cards presented with plain-language descriptions:

Prescriptive base (Sections 9.36.2–9.36.4) — meets Tier 1 only, insufficient for Manitoba. Shown for reference.

Tiered Prescriptive (Section 9.36.9) — earn conservation points above base minimums. Recommended badge. Most users land here.

Performance (Section 9.36.7) — energy modelling path. Screening tool only.

Tiered Prescriptive Path — Step 4: Base Requirements

User must pass base code minimums (Table 9.36.2.6.-B for buildings with HRV) before earning any points. Each component shows a pass/fail indicator in real time.

| Component | Input | Code Reference | Zone 7A Minimum |
| --- | --- | --- | --- |
| Above-grade walls | Effective RSI | Table 9.36.2.6.-B | RSI 2.97 |
| Ceilings below attics | Effective RSI | Table 9.36.2.6.-B | RSI 8.67 |
| Cathedral ceilings / flat roofs | Effective RSI | Table 9.36.2.6.-B | RSI 5.02 |
| Floors over unheated spaces | Effective RSI | Table 9.36.2.6.-B | RSI 5.02 |
| Windows & doors | U-value or ER | Table 9.36.2.7.-A | U ≤1.61 / ER ≥25 |
| Foundation walls | Effective RSI | Table 9.36.2.8.-B | RSI 2.98 |
| Slabs-on-grade | Effective RSI | Table 9.36.2.8.-B | RSI 2.84 |

Tiered Prescriptive Path — Step 5: Earn Conservation Points

Accordion interface — one category at a time. Each category shows the user's input, the points earned at that value for their zone, and the running total. Live point tracker at top of screen throughout: X / 20 total, X / 5 envelope.

Categories in recommended order (highest-impact first for Zone 7A):

| # | Category | Code Reference | Max pts Zone 7A | Notes |
| --- | --- | --- | --- | --- |
| 1 | Space heating — cold-climate ASHP | 9.36.9.9 | ~35 | Biggest single earner. Flags heat pump as recommended path. |
| 2 | Space heating — standard ASHP | 9.36.9.9 | ~23 | Still very high. Shown if cold-climate not selected. |
| 3 | Airtightness | 9.36.9.8 | ~15 | Requires blower door test. Optional but high value. |
| 4 | Above-grade walls | 9.36.9.5 | ~12 | Envelope points. Counted toward 5-pt envelope minimum. |
| 5 | Windows & doors | 9.36.9.6 | ~13 | High potential. Envelope points. |
| 6 | HRV/ERV efficiency | 9.36.9.9 | ~5 | Prerequisite already met; points for exceeding minimum SRE. |
| 7 | Ceilings below attics | 9.36.9.5 | ~1.2 | Envelope points. |
| 8 | Cathedral ceilings / flat roofs | 9.36.9.5 | ~1.8 | Envelope points. |
| 9 | Foundation walls | 9.36.9.7 | ~3.2 | Envelope points. |
| 10 | Service water heating | 9.36.9.10 | ~3.1 | Water heater efficiency. |
| 11 | Drain-water heat recovery | 9.36.9.10 | ~4.2 | High efficiency drain units. |
| 12 | Gas/propane furnace | 9.36.9.9 | ~1.4 | AFUE 96–98%. Shown only if heat pump not selected. |
| 13 | Exposed floors | 9.36.9.5 | ~1.1 | Envelope points. Only if ≥30 m². |
| 14 | Building volume | 9.36.9.11 | 10 | If each unit ≤230 m³, or total ≤390 m³. |

Tiered Prescriptive Path — Step 6: Results

Tier achieved displayed prominently (Tier 1 through 5, Manitoba minimum is Tier 3)

Component summary table: each input, points earned, pass/fail

Gap analysis: if not at Tier 3, shows "what-if" suggestions — e.g. "Upgrade walls from RSI 3.85 to RSI 4.29 → +2.2 points"

Tier 3 envelope minimum check: if total points ≥20 but envelope points <5, flags this specifically

PDF export: one-page compliance summary formatted for permit submission

Save project: email-based, no account required — user gets a link to return and edit

Performance Screening Path

Simplified energy screening based on key inputs. Output is an estimated percent improvement over the NBC reference house and likely tier. Not permit-ready — clearly labelled as a screening estimate throughout.

Inputs: location, building volume, wall/roof/window specs (pre-filled if base check was done), heating system, hot water, airtightness if known

Output: estimated % improvement range, likely tier, clear disclaimer, link to energy modeller directory (ties to page 2C.2)

Code reference: NBC 2025 Table 9.36.7.2 (performance tier thresholds)

5. COMMERCIAL TOOL FLOW

NECB 2025 — Manitoba adopts Tier 2 (≤75% of reference building energy, or ≥25% improvement).

Steps 1–2 — Location and Building Basics

City dropdown → climate zone (7A, 7B, 8)

Building occupancy type (office, retail, school, warehouse, etc.)

Number of storeys, gross floor area (m²)

Conditioned or semi-heated designation

Step 3 — Compliance Path

Prescriptive: check each system independently against NECB tables. All five parts (3–7) must comply.

Trade-off: envelope trade-offs within category. Cannot trade across systems (e.g. envelope vs lighting).

Performance: screening estimator (≥25% improvement over reference building). Refer to certified modeller for permits.

Prescriptive Path — Five Sections

Part 3 — Building Envelope

| Check | Input | Code Reference | Zone 7A Limit |
| --- | --- | --- | --- |
| FDWR calculation | Fenestration + door area / gross wall area | NECB 3.2.1.4 | ≤40% (formula-adjusted for HDD) |
| Above-grade walls | U-value W/(m²·K) | Table 3.2.2.2 | ≤0.215 |
| Roofs | U-value W/(m²·K) | Table 3.2.2.2 | ≤0.121 |
| Floors | U-value W/(m²·K) | Table 3.2.2.2 | ≤0.138 |
| Vertical fenestration | U-value W/(m²·K) | Table 3.2.2.3 | ≤1.73 |
| Skylights | U-value W/(m²·K) | Table 3.2.2.3 | ≤2.41 |
| Doors | U-value W/(m²·K) | Table 3.2.2.4 | ≤1.90 |
| Below-grade walls | U-value W/(m²·K) | Table 3.2.3.1 | ≤0.284 |
| Air barrier | Confirm continuous system | NECB 3.2.4.1 | Required |

Part 4 — Lighting

Space type selector → auto-fills LPD limit from NECB Table 4.2.1.5

User inputs installed LPD (W/m²) → pass/fail

Lighting controls checklist: occupancy sensors, automatic daylight controls

Part 5 — HVAC

System type selector

Energy recovery ventilation check: fan airflow vs threshold (Tables 5.2.10.1-A/B)

Equipment efficiency inputs vs minimums (Tables 5.2.12.1 series)

Balancing requirements confirmation

Part 6 — Service Water Heating

Equipment type + efficiency rating → pass/fail vs NECB minimums

Part 7 — Electrical

Motor sizes → minimum efficiency check

Power factor requirements

Trade-off Path (Envelope Only)

NECB Section 3.3 permits trade-offs within the envelope category only. A component that exceeds the prescriptive minimum can compensate for one that falls below, provided absolute minimums are not violated.

User inputs all above-ground assembly U-values and areas

Tool calculates area-weighted average thermal transmittance

Compares against reference building thermal transmittance

Flags components below prescriptive minimums and calculates required compensation

FDWR check still applies — cannot be traded away

Performance Screening Path (Commercial)

Simplified inputs: location, envelope specs, lighting LPD, HVAC type, hot water

Estimated % improvement vs NECB reference building

Tier 2 threshold: ≤75% of reference building energy (≥25% improvement)

Output: estimated range, likely tier, disclaimer, referral to energy modellers

Code reference: NECB 2025 Table 10.1.2.1

6. CODE REFERENCE STRATEGY

Every data point, threshold, and calculation in the tool is traceable to a specific code table or article. References are presented at three levels:

Level 1 — Inline Citation

Every pass/fail threshold and point value shows a linked citation directly in the UI:

Example  "Above-grade walls — Zone 7A: maximum U-value 0.215 W/(m²·K)  [NECB 2025 Table 3.2.2.2 ↗]"
Clicking the link opens the relevant page of the hosted PDF.

Level 2 — Site Content Links

Where a KnowYourCodesMB.ca content page already covers the topic (most topics — 25 pages were drafted in Phase 1), the tool links to that page first. The PDF is a secondary "see source" link. This keeps users in the site ecosystem and reinforces the content investment.

Level 3 — Expandable Code Text

For key articles and tables, the relevant code text is available inline — collapsed by default, expandable with a "View code text" toggle. This allows users to read the exact wording without leaving the tool. EVS to review presentation of code language for accuracy.

PDF Hosting

The NRC does not publish stable anchor URLs for code articles. PDFs are hosted on KnowYourCodesMB.ca at stable paths:

knowyourcodes.ca/codes/NBC-2025-Vol2.pdf

knowyourcodes.ca/codes/NECB-2025.pdf

Tool links use PDF page anchors, e.g. NBC-2025-Vol2.pdf#page=294 to jump directly to the relevant page.

7. OUTPUT AND SAVE FEATURES

On-Screen Results

Live, reactive — updates as user inputs change

Component-by-component summary with pass/fail indicators

Tier achieved displayed prominently

Gap analysis and what-if suggestions when not at required tier

PDF Export

One-page compliance summary per project

Designed to be attachable to a permit application

Includes: project address, date, climate zone, path chosen, all component values, tier/compliance result

Tool version number and disclaimer included

Residential: includes code references per component

Commercial: separate section per NECB part

Save / Share

Email-based — no account or password required

User enters email address, receives a unique link to their saved project

Link allows returning to edit inputs and re-run calculations

Projects are not publicly accessible — link is private

Projects auto-expire after 12 months (or configurable)

8. DATA FOUNDATION

The underlying code data has already been structured and validated as part of this project. All values have been extracted directly from the 2025 code books.

| Dataset | Status | Source |
| --- | --- | --- |
| 24 Manitoba cities → HDD + climate zone | Complete | NECB/NBC Appendix C |
| NECB 2025 envelope tables (Zones 7A/7B/8) | Complete | NECB 2025 Tables 3.2.2.2–3.2.3.1 |
| NBC 2025 base prescriptive tables (with/without HRV) | Complete | NBC 2025 Tables 9.36.2.6-A/B, 9.36.2.7-A, 9.36.2.8-A/B |
| NBC 2025 tiered point tables (all 15 categories) | Complete | NBC 2025 Tables 9.36.9.5–9.36.9.11 |
| Tier thresholds (residential + commercial) | Complete | NBC Table 9.36.9.2, NECB Table 10.1.2.1 |
| Performance path tier targets | Complete | NBC Table 9.36.7.2 |
| NECB lighting LPD tables (Part 4) | Pending | NECB 2025 Table 4.2.1.5 — extractable from PDF |
| NECB HVAC equipment efficiency tables (Part 5) | Pending | NECB 2025 Tables 5.2.12.1 series — extractable from PDF |
| Manitoba energy code amendments | Complete | Manitoba Regulation 79/2023 |

All data is stored in a structured JSON file (mb-code-data.json) that serves as the tool's calculation engine. Pending items can be extracted from the NECB 2025 PDF before build begins.

9. TECHNICAL APPROACH

The tool is built as an addition to the KnowYourCodesMB.ca website, using the same technology stack.

| Component | Technology | Notes |
| --- | --- | --- |
| Frontend framework | Next.js (React) | Same stack as site |
| Database | PostgreSQL | Saved projects, usage analytics |
| Calculation engine | JSON data + server-side TypeScript | All code table data pre-structured |
| PDF export | React-PDF or Puppeteer | Server-side generation |
| PDF hosting | Static file hosting | NBC and NECB 2025 PDFs at stable URLs |
| Save / share | Email token + serialized JSON | No account system required |
| Code references | PDF page anchors + internal links | Linked from every threshold value |

Complexity Assessment

| Feature | Complexity | Notes |
| --- | --- | --- |
| Residential tiered prescriptive | Medium | Point lookup + running tally + gap analysis |
| Residential performance screening | Medium | Simplified heat loss model |
| Residential trade-off | Low-Medium | Simple envelope trade-off per NBC 9.36.2.11 |
| Commercial prescriptive | Medium-High | Five independent sections, more tables |
| Commercial trade-off | High | Weighted average calculation + compensation logic |
| Commercial performance screening | Medium | Simplified model, same pattern as residential |
| PDF export | Low-Medium | Templated, server-side |
| Save / share (email token) | Low | Standard pattern |
| Code reference links | Low | Data already tagged with table references |
| Role-adapted UX | Low-Medium | Language/detail level switching, same logic |

10. OPEN ITEMS

The following items require input from EngGeoMB or EVS before or during build.

| # | Item | Owner | Impact |
| --- | --- | --- | --- |
| 1 | NECB 2025 lighting LPD table (Table 4.2.1.5) — needs to be extracted and structured. Data is in the NECB 2025 PDF. | Honest Agency | Required for commercial lighting check |
| 2 | NECB 2025 HVAC equipment efficiency tables (Tables 5.2.12.1 series) — extensive but extractable. | Honest Agency | Required for commercial HVAC check |
| 3 | GHG performance level alongside NECB Tier 2 (Part 11) — not yet announced by Government of Manitoba. | EngGeoMB / GoM | May add a GHG check to commercial tool |
| 4 | Manitoba climate zone list — confirm HDD values for any cities not in the 24 already mapped. | EVS / EngGeoMB | Required for location selector |
| 5 | Presentation of inline code text — EVS to review how code language is displayed in the tool. | EVS | Required before launch |
| 6 | PDF hosting arrangement — confirm domain structure and file naming for hosted PDFs. | EngGeoMB | Required for code reference links |
| 7 | Interpretation Notes governance — who approves notes added to the tool over time. | EngGeoMB | Required for ongoing maintenance |

11. WHAT'S OUT OF SCOPE

The following are explicitly not included in Phase 1 of the tool:

Full HOT2000 energy modelling (performance path is screening only)

Alterations and renovations to existing buildings (NECB Part 13 / NBC Section 9.36.14) — complex rules, low traffic, Phase 2 candidate

On-reserve jurisdiction (jurisdictional complexity, requires co-development)

User accounts and project management beyond email-based save/share

Bilingual (French) version of the tool — Phase 2 if required

Integration with permit systems or authority having jurisdiction (AHJ) workflows

Automated code updates — data will need manual review and update when code amendments are issued

12. RECOMMENDED NEXT STEPS

1. EngGeoMB and EVS review and approve this scope document

2. Resolve open items 3–7 (items 1–2 can be handled by Honest Agency in parallel)

3. Extract remaining NECB data (lighting LPD and HVAC efficiency tables)

4. Wireframe the residential tiered prescriptive flow for EngGeoMB review before build begins

5. Begin build — residential tool first, commercial second

6. EVS technical review of tool outputs against code before launch